| # FUNKTION ONE RESEARCH |  |  |  |
| --- | --- | --- | --- |
|  |  | DATE | 17th Dec 08 |

|  | Powered Res2 Presets Dec 2008 |
| --- | --- |
| P0 | Extra Bass High Pass at 28Hz, Bass 0dB, HF -10dB |
| P1 | Normal High Pass at 28Hz, Bass 0dB, HF -8dB |
| P2 | Less Bass or 2 a side High Pass at 28Hz, Bass 0dB, HF -6dB (normal or maybe when 2 a side, bass coupled) |
| P3 | Two a side High Pass at 28Hz, Bass 0dB, HF -4dB |
| P4 | With Sub High Pass at 70Hz, Bass 0dB, HF -7dB (Use with F218 or Infrabass at 60 – 70Hz Lowpass) |
| P5 | DJ Mons High Pass at 28Hz, HF -7dB, HF Phase Rev and No Delay (Full Range Use for DJ Monitors if they don’t like the mid-hi time delay..) |
| P6 | DJ Mons Heavy, with extra bass High Pass at 28Hz, HF -7dB, HF Phase Rev and No Delay (Full Range Use for DJ Monitors if they don’t like the mid-hi time delay..) |
| P7 | DJ Mons+ Sub High Pass at 70Hz, HF -7dB, HF Phase Rev and No Delay (Use with F218, F221 or Infrabass at 60 or 70Hz Lowpass) (Useful for DJ Monitors if they don’t like the mid-hi time delay..) |
| P8 | Muted |
| P9 | Muted |